Caracas, January 30 1993.

If you are using a Macintosh set Font to Courier 12

ENTHALPY OF FORMATION TABLES FOR THE 48 v1.0

-------- -- --------- ------ --- --- --

Hello, my name is Gustavo Delfino. I am a mechanical engineering student at Universidad Metropolitana in Caracas, Venezuela. Here is a contribution to all thermodynamics students. This electronic tables are made by the interpolation of cubic splines. Download using text mode in Kermit AND HP48 use translate code 3. This program requires about 8000 bytes of free RAM. After download you will find the following programs inside a directory:

1  N2:   Nitrogen, Diatomic

2  N:    Nitrogen, Monoatomic

3  O2:   Oxygen, Diatomic

4  O:    Oxygen, Monoatomic

5  C02:  Carbon Dioxide

6  CO:   Carbon Monoxide

7  H2O:  Water (gas)

8  LH2O: Water (liquid)

9  OH:   Hydroxyl

10 H2:   Hydrogen, Diatomic

11 H:    Hydrogen, Monoatomic

12 NO:   Nitric Oxide

13 NO2:  Nitric Dioxide

14 ENTALFORM: Splines evaluator DON'T TOUCH

15 EFData:    Splines coefficients DON'T TOUCH

Programs 1-13 are functions. You give the temperature (Kelvin) and you get the enthalpy of formation. Example:

If you want to know the enthalpy of formation (ef) of Carbon Dioxide at T=2500_K you normally should have to search in a book for the ef of CO2 at 298_K and search for the increment at 2500_K, this is -393522 + 121926 = -271596_kJ/kgmol; using the programs you just type the temperature (Kelvin) and press CO2 at the menu bar to obtain -271596.2.

As you see, the error is small and no interpolation is required. But the better thing of this tables is that YOU CAN SOLVE EQUATIONS USING THE TEMPERATURE AS THE UNKNOWN VARIABLE, this is great to calculate the adiabatic flame temperature. Here is an example:

Calculate the adiabatic flame temperature from the following combustion reaction:

C8H18   + 12.5*02+ 47*N2 -----> 8*CO2 + 9*H2O + 47*N2

liq

as the enthalpy is constant Hprod = Hreact

from tables enthalpy of C8H18(liq) is -249952_kJ/kgmol so we write the following equation  in the calculator:

'-249952=8*CO2(T)+9*H2O(T)+47*N2(T)'

then press SOLVE STEQ SOLVR then give an initial approximation of T: 2000 T (press T on menu bar) RED (left arrow) T and wait for the solution T=2394.5_K.

The valid temperature range is 0<=T<=6000_K. If you exceed this range you will still get an answer (probably wrong) but you will see "FUERA DE RANGO" on the screen. The temperature nodes taken for the interpolation were: 0 100 200 298 300 400 600 800 1200 1600 2000 2400 3000 4000 5000 6000_K. The interpolation was made by my own multiple-interpolation program. (coming this year).

If you like this program or anything, please let me kwon. My number is 73000,1127.

Bye.

Bibliography:

Fundamentals of Classical Thermodynamics

3rd Edition

Gordon J. Van Wylen

Richard E. Sonntag

pages 708-719.