//////////////////////////////////////////////////////////////////////

//                                                                  //

//  IMPORTANT: THIS MANUAL REFERS TO THE FULL VERSION OF IRPLUS!!!  //

//                                                                  //

//////////////////////////////////////////////////////////////////////

IRPLUS

======

Copyright (c) 1994 J.Goldin

Jacob's Chemical Associated

P.O.Box 350099

Brooklyn, NY 11235

CIS: 73441,351

CONTENTS

========

1. Introduction

2. Installation

3. Working with IRPLUS

STARTING IRPLUS

OPENING A SPECTRUM'S FILE AND USING FILE MENU COMMANDS

OPENING A LIBRARY SPECTRUM AND USING LIBRARY MENU COMMANDS

MANIPULATING SPECTRA AND USING SPECTRA SCREEN COMMANDS

SEARCH

PRINT

SCREEN SAVER

4. System requirements

1. Introduction

===============

IRPLUS is an ultimate IR and UV-VIS spectroscopy research tool

for DOS environment. It has an array of unique features that

cannot be found anywhere else. It gives you:

-- possibility to manipulate as many as 10 IR and/or UV and/or FL

spectra at the same time;

-- preview of a spectrum before it is opened;

-- a sophisticated search algorithm with the 10 best active

matching spectra and a list of 40 more matches;

-- possibility to search for matching spectra through PE and JCAMP

4.24 spectra's files and PE libraries of spectra;

-- possibility to search for a particular matching peak, a number

of peaks, and/or areas (up to 10);

-- direct access to and full control over PE libraries of spectra

including editing and deleting library entries;

-- possibility to convert PE library spectra to PE file format or

to JCAMP 4.24 format.

Other features include:

-- complete operating shell with all file and directory maintanance

functions;

-- transmittance/absorbance mode conversion;

-- adding and subtracting spectra;

-- zooming spectra and displaying peaks;

-- changing view ranges which allows switching from IR to UV or FL

mode;

-- printing spectra on plain or chart paper using HP compatible

laser or inkjet printers;

-- the program treats PE spectra's files and JCAMP 4.24 files the

same way, no conversion needed (although it is available);

-- context sensitive help;

-- built in screen saver going off in 4 minutes if no keys or mouse

buttons are pressed.

2. Installation

===============

Insert the installation diskette into the drive A: or B: and

type "A:install" or "B:install" respectively. You will be

prompted to type in the name of the directory you want to

install IRPLUS to (or just hit ENTER to leave it C:\IRPLUS by

default). After you see the message "INSTALLATION IS

SUCCESSFULLY COMPLETED", hit ESC to exit the installation

program.

3. Working with IRPLUS

======================

STARTING IRPLUS

---------------

After the installation make IRPLUS directory your working

directory. Type "irplus" to start the program. You will see the

title screen. To exit the screen, press ESC.

You can quit IRPLUS at any time (except when you see the help

or title screens) by pressing CTRL-C.

OPENING A SPECTRUM'S FILE AND USING FILE MENU COMMANDS

------------------------------------------------------

The file menu screen will appear. This will reflect the

directory structure of your computer. The blue menu buttons on

the bottom of the screen reflect the set of commands you can

issue from here. To issue a command, you can either press a

functional key or click on the corresponding menu button with

the mouse. Open the directory containing spectra's files by

highlighting it with the arrow keys or mouse cursor and press

ENTER or double-click the left mouse button ("SAMPLES" directory

comes with the installation diskette). The content of the

directory will appear on the screen. Spectra's files and

libraries will have respective designations. When you then

highlight a spectrum's file (extension .SP for PE spectrum or

.DX for JCAMP spectrum), the preview of the spectrum will appear.

Here is the list of commands you can perform from here with

short explanations when needed:

-- Help  - F1.

-- Show previously found matches - SHIFT-F1. It only works if any

search has been done before.

-- Preview spectrum screen - F3. Works only with spectra's files.

Point to a spectrum's file with arrow keys or mouse and press

F3. If at least one spectrum has been already opened, you will

see what the spectra screen is going to look like.

-- Edit spectra comment - F4. Works only with spectra's files.

Point to a spectrum's file with arrow keys or mouse and press

F4. You will be prompted to enter a new spectrum's name or press

ESC to cancel.

-- Copy file - F5. Works with all types of files. Point to a file

with arrow keys or mouse and press F5. If you do not type

extension of the destination file, it will be the same as the

source file's extension. If you do not want any extension,

simply press space bar.

-- Move/Rename file - F6. Works with all types of files. Point to a

file with arrow keys or mouse and press F6. You will be prompted

to type in a new file name.

-- Create new subdirectory - F7. You  will be prompted to type in a

new directory name.

-- Delete file or directory - F8. Works with archive files and

directories. Point to a file or directory with arrow keys or

mouse and press F8. You will be asked for the confirmation.

-- View spectrum comments - F9.  This will take effect only if

there are any spectra's files in the directory. This may take

some time. You will see the same menu,  but instead of

descriptions "PE spectrum" or "JCAMP spectrum" there will be

spectra's comments.

-- Convert data file to another format - F10. Works only with

spectra's files. The conversion menu will appear, and you will

be prompted to type in a name of a converted file.

-- Go to the spectra screen - ESC or ENTER.

-- Run a program from within this program (this command can only be

accessed  from the keyboard):

if the program file is highlighted - ENTER or double-click left

mouse button;

if the program file is not highlighted - Ctrl-ENTER. You will

be prompted to type in the program's name.

-- Browse current directory - PgUp & PgDn.

-- Open a spectrum - point & ENTER or double-click left mouse

button.

-- Exit to DOS - Ctrl-C.

To open the spectrum's file, hit ENTER or double-click the

left mouse button. The spectra screen will appear.

Now return to the file menu by pressing ESC or clicking the

right mouse button for further instructions.

OPENING A LIBRARY SPECTRUM AND USING LIBRARY MENU COMMANDS

----------------------------------------------------------

When you are in the file menu showing any libraries of spectra

(extension .DLB), highlight a library file (it should have the

designation * LIBRARY * to the left) and open it - press ENTER

or double-click on it with the mouse left button.

The following is the list of commands you can perform from here

with short explanations when needed:

-- Help - F1.

-- Show found matches - SHIFT-F1. It only works if any search has

been done before.

-- Preview of the spectra screen - F3. Point to a spectrum's

comment with arrow keys or mouse and press F3. If at least one

spectrum has been already opened, you will see what the spectra

screen is going to look like.

-- Edit library spectrum comment - F4. Point to a spectrum's

comment with arrow  keys or mouse and press F4. You will be

prompted to enter a new comment or press ESC to exit.

-- Delete library spectra - F8. You will be asked for the

confirmation.

-- Extract a spectrum from the library to a spectrum's file - F10.

You will be prompted to type in a new file's name.

-- Browse library - PgUp and PgDn.

-- Open a library spectrum - ENTER or double-click the left mouse

button.

-- Go to the file menu screen - ESC or click the right mouse button.

-- Exit to DOS - Ctrl-C.

MANIPULATING SPECTRA AND USING SPECTRA SCREEN COMMANDS

------------------------------------------------------

Open a library spectrum. The spectra screen will appear.

You can manipulate the spectra on the screen using functional

keys or by clicking on menu buttons on the bottom of the screen.

Every functional key has its mouse equivalent (except for

SHIFT-F10 -- run the configuration utility). To go to the next

set of buttons, press GRAY PLUS on the right side of the

keyboard or click the rightmost blue menu button on the bottom

of the screen.

The spectrum that was opened last is the active spectrum (You

can switch active spectrum if you wish). It is shown in solid

line and its name appears in the right upper corner of the

screen. Some commands from the menu on the bottom of the screen

affect only this spectrum, the others -- all spectra.

The following is the list of commands you can perform from here

with short explanations when needed:

-- Accessing help

HELP - F1.

-- Manipulating spectra

Horizontal shift (All spectra) Left & Right arrows.

Vertical shift:

Active spectrum - Up & Down arrows;

All spectra - SHIFT PgUp & SHIFT PgDn.

Vertical expansion & reduction:

Active spectrum - SHIFT Up & SHIFT Down;

All spectra - SHIFT Home & SHIFT End.

-- Spectral calculations

Add all spectra on the screen (not more than 7) - "+".

Subtract spectra from the active spectrum (not more than 7) "-".

NOTES:

1.These functions allow you to add up to 7 spectra on

the screen or subtract up to 6 spectra from the active one. They

work on the What You See Is What You Get philosophy. If a

spectrum is moved or modified, the result will be different.

2. Unless deleted, the previous resulting spectrum will be

calculated in with the other spectra on the screen.

-- View options

Switch absorbance/transmittance mode - F2.

Show only the active spectrum and hide the rest - F3.

Hide/Show the active spectrum - SHIFT-F3.

Switch the active spectrum:

Forward - F4;

Backward - SHIFT-F4.

Erase the active spectrum from the screen - F9.

Erase all spectra from the screen - SHIFT-F9.

Zoom in (for mouse users only) - point and drag.

NOTES:

1. To pinpoint exact value of transmittance or absorbance,

position the mouse cursor on any point on the spectra screen and

click once. Values are organized by color and are displayed for

all spectra below the specta window.

2. If, on accident, the mouse is dragged slightly instead of

just a click you will see extreme magnification. Press ESC or

the right mouse button to return to the normal screen.

Zoom out (for mouse users only) - ESC or click the right mouse

button.

Refresh screen - F10.

Vertical cursor - will display a dark gray vertical line across

the spectra window

Show/Hide vertical cursor - F5.

NOTE: To move the cursor, use cursor keys or point the mouse to the

desirable location and click its left button.

-- Peaks

Show/Hide the active spectrum's peaks - F6. Click once on the

spectra screen and the vertical line will find the nearest peak

from the left of the active spectra.

Show/Hide all peaks - SHIFT-F6. Click once on the spectra

screen an the vertical line will find the nearest peak from the

left of any spectra shown. The color of the wavenumber reading

will match the color of the spectrum with the particular peak.

To move to the next peak, use cursor keys or point the mouse to

the desirable location and click its left mouse button.

-- Search (will be considered in more details later)

Start search - F7.

-- Show previously found matches - SHIFT-F1.

-- Print (will be considered in more details later)

-- Print a spectrum (HP compatible printers) - F8.

-- Change view ranges and the search tolerance (This command cannot

be accessed by mouse)

-- Run configuration utility - SHIFT-F10.

-- Exit spectra screen

-- Go to file system menu (when zoom is off) - Enter or ESC or

click the right mouse button (If there are 11 spectra on the

screen, one or more spectra have to be erased first).

-- Exit to DOS - Ctrl-C

SEARCH

------

If you hit F7 to start search, only the active spectrum will

stay on the screen. Vertical cursor will appear, and you will be

prompted to point to the areas where you want to perform the

search. If you want the default margins (this is what you see),

just press F10, ESC or the right mouse button. Otherwise, enter

the left margin for the first area. To do that, move the

vertical cursor with arrow keys or by clicking the left mouse

button and press ENTER or  the blue menu button. Then repeat for

the right margin, and so on. You can enter up to 10 contingent

areas. At any moment, you can go to the search menu by pressing

F10, ESC or the right mouse button.

After you chose the margin, you see the search menu:

SPECTRA'S FILES

LIBRARY SPECTRA

BOTH FILES & LIBRARIES

BROWSE DIRECTORIES

QUIT SEARCH

and the corresponding menu on the bottom of the screen.

There can be the following possibilities:

-- you chose "SPECTRA'S FILES" but there are no such files in the

current directory. The search will be canceled;

-- you chose "LIBRARY SPECTRA" but there are no libraries in the

current directory. You will be prompted to change drive or

directory until any library is found or you press ESC or the

right mouse button;

If you choose "BROWSE DIRECTORIES", you can change the current

directory until you press F10 (search here), ESC or the right

mouse button (cancel the search).

PRINT

-----

If you pressed F8, the print menu appears on the bottom of the

screen:

F2 (Chart to file) - the screen will be formatted for chart

paper and sent to a file;

F3 (Chart to HP Lj) - the screen will be formatted for chart

paper and sent to printer;

F4 (Plain to file) - the screen will be sent to a file as you

see it;

F5 (Plain to HP Lj) - the screen will be sent to printer as you

see it.

ESC (Cancel print) - go to the spectra screen.

SCREEN SAVER

------------

Do not be scared if when you come back in more than 4 minutes

you see the screen all messed up. Just press any key or move the mouse

4. System requirements

======================

IBM compatible 286 and up

Hard disk with at least 500K of free disk space

At least 640K of RAM

VGA or better color monitor

DOS 3.3 or above

Optional - mouse or other pointing device

Optional - HP Deskjet or HP Laserjet